28 PRODUCTION SYSTEMS AND PROCESS PLANNING

# Review Questions

- Provide a definition of production systems?

Answer. Production systems consist of people, equipment, and procedures designed for the combination of materials and processes that constitute a firm’s manufacturing operations.

- What are the two categories of production systems?

Answer. According to the text, the two categories are (1) production facilities and (2) manufacturing support systems. Production facilities refer to the physical equipment and the arrangement of equipment in the factory. Manufacturing support systems are the procedures used by the company to manage production and solve the technical and logistics problems encountered in ordering materials, moving work through the factory, and ensuring that products meet quality standards.

- Name the four types of plant layouts typically found in factories. What types of production are each normally associated with?

Answer. The four types of plant layout are (1) fixed position layout, normally associated with low production of large products, (2) process layout, usually associated with low- and medium-quantities of batches, (3) cellular layout, associated with medium-quantity production of part or product families, and (4) product layout, associated with high production of complex assembled products.

- Define manufacturing support systems.

Answer. Manufacturing support systems are the set of procedures and systems used by a company to solve the technical and logistics problems encountered in planning the processes, ordering materials, controlling production, and ensuring that the company’s products meet required quality specifications.

- What is process planning?

- Answer. Process planning consists of determining the most appropriate manufacturing processes and the order in which they should be performed to produce a given part or product according to the specifications established by design engineering. If it is an assembled product, process planning includes deciding the appropriate sequence of assembly steps.

- Define manufacturing engineering.

- Answer. Manufacturing engineering is a technical staff department responsible for planning the manufacturing processes for a product and its components. Its scope is broader than process planning because it includes areas such as solving problems in production and continuous improvement.

- Identify some of the details and decisions that are included within the scope of process planning.

- Answer. Table 28.1 lists the following details and decisions in process planning: (1) the processes required and the sequence in which they are performed; (2) equipment selection; (3) tools, dies, molds, fixtures, and gages that will be needed; (4) cutting tools and cutting conditions for machining operations; (5) methods for manual operations (e.g., assembly) and manual portions of machine cycles (e.g., loading and unloading a production machine); (6) time standards for each operation; and (7) estimates of production costs. Not all of these details and decisions are the exclusive responsibility of the manufacturing engineering department.

- What is a route sheet?

- Answer. A route sheet is a listing of the production operations and their sequence required to make a given part. It also lists the equipment and special tooling that is required.

- What is the difference between a basic process and a secondary process?

- Answer. A basic process establishes the starting geometry of the work material; e.g., casting, rolled sheet metal, drawn metal bar.  Secondary processes are used to refine the starting geometry and transform the material into final shape and size; machining and sheet metal pressworking are common secondary processes.

- What is a precedence constraint in process planning?

- Answer. A precedence constraint is a limitation on the order in which processing or assembly operations can be performed on a given workpart; for example, a hole must be drilled before it can be tapped. The definition appears in Table 28.3.

- In the make or buy decision, why is it that purchasing a component from a vendor may cost more than producing the component internally, even though the quoted price from the vendor is lower than the internal price?

- Answer. Because purchasing the component may cause idle equipment and staff in the factory for which the company must still pay.

- Identify some of the important factors that should enter into the make or buy decision.

- Answer. The factors in the make or buy decision include (1) cost, (2) whether the process is available internally, (3) production quantity, (4) product life, (5) whether or not the component is a standard commercially available hardware item, (6) supplier reliability, and (7) possible need for alternative sources.

- Name three of the general principles and guidelines in design for manufacturability.

- Answer. Table 28.5 lists the following DFM principles and guidelines: (1) Minimize number of components. (2) Use standard commercially available components. (3) Use common parts across product lines. (4) Design for ease of part fabrication. (5) Design parts with tolerances that are within process capability. (6) Design the product to be foolproof during assembly. (7) Minimize flexible components. (8) Design for ease of assembly. (9) Use modular design. (10) Shape parts and products for ease of packaging. (11) Eliminate or reduce adjustment required.

- What is concurrent engineering?

- Answer. Concurrent engineering is an approach used by companies to reduce the time to bring a new product to market. Manufacturing engineering becomes involved early in the product development cycle. In addition, other functions are also involved, such as field service, quality engineering, the manufacturing departments, vendors supplying critical components, and in some cases customers who will use the product.