2 ENGINEERING MATERIALS

Review Questions

- What are some of the general properties that distinguish metals from ceramics and polymers?

- Answer. Typical metallic properties include: high strength and stiffness, good electrical and thermal conductivity, and higher density than ceramics or polymers.

- What are the two major groups of metals?  Define them.

- Answer. Ferrous metals, which are based on iron; and nonferrous, which includes all others.

- What is an alloy?

- Answer. An alloy is a metal comprised of two or more elements, at least one of which is metallic.

- What is the range of carbon percentages that defines an iron-carbon alloy as a steel?

- Answer. The carbon content ranges from 0.02% to 2.11%.

- What is the range of carbon percentages that defines an iron-carbon alloy as cast iron?

- Answer. The carbon content ranges from 2.11% to about 5%.

- Identify some of the common alloying elements other than carbon in low alloy steels.

- Answer. The common alloying elements in low alloy steel are Cr, Mn, Mo, Ni, and V.

- What is the predominant alloying element in all of the stainless steels?

- Answer. Chromium.

- Why is austenitic stainless steel called by that name?

- Answer. It is called austenitic because this alloy exists in its austenitic phase at room temperature.  The reason is that nickel has the effect of enlarging the austenitic temperature range to include room temperature.

- Besides high carbon content, what other alloying element is characteristic of the cast irons?

- Answer. Silicon.

- Identify some of the properties for which aluminum is noted?

- Answer. Aluminum is noted for its low density, high electrical and thermal conductivity, formability, good corrosion resistance due to the formation of a tough oxide film on its surface, and ability to be alloyed and strengthened to achieve good strength-to-weight ratios.

- What are some of the noteworthy properties of magnesium?

- Answer. Magnesium is noted for its very low density (lightest of the structural metals), propensity to oxidize (which can cause problems in processing), and low strength; however, it can be alloyed and strengthened by methods similar to those used for aluminum alloys to achieve respectable strength-to-weight ratios.

- What is the most important engineering property of copper that determines most of its applications?

- Answer. Its high electrical conductivity (low resistivity).

- What elements are traditionally alloyed with copper to form (a) bronze and (b) brass?

- Answer. The elements are (a) tin and (b) zinc, respectivley.

- What are some of the important applications of nickel?

- Answer. The important applications of Ni are (1) as an alloying ingredient in steel, e.g., stainless steel; (2) for plating of steel to resist corrosion; and (3) to form nickel-based alloys noted for high-temperature performance and corrosion resistance.

- What are the noteworthy properties of titanium?

- Answer. Titanium is noted for its high strength-to-weight ratio, corrosion resistance (due to the formation of a thin but tough oxide film), and high temperature strength.

- Identify some of the important applications of zinc.

- Answer. The important applications of Zn are (1) die castings - zinc is an easy metal to cast; (2) as a coating in galvanized steel; (3) as an alloying element with copper to form brass.

- The superalloys divide into three basic groups, according to the base metal used in the alloy.  Name the three groups.

- Answer. The three groups are (1) iron-based alloys, (2) nickel-based alloys, and (3) cobalt-based alloys.

- What is so special about the superalloys?  What distinguishes them from other alloys?

- Answer. The superalloys are generally distinguished by their strength and resistance to corrosion and oxidation at elevated temperatures.

- What is a ceramic?

- Answer. A ceramic is an inorganic compound, consisting of a metal (or semi-metal) and one or more non-metals.

- What is the difference between the traditional ceramics and the new ceramics?

- Answer. Traditional ceramics are based primarily on clay products (e.g., pottery, bricks) while new ceramics are more recently developed ceramics which are generally simpler in chemical composition (e.g., oxides, carbides).

- What is the feature that distinguishes glass from the traditional and new ceramics?

- Answer. Glass is noncrystalline (amorphous), while most other ceramics assume a crystalline structure.

- What are the general mechanical properties of ceramic materials?

- Answer. Typical mechanical properties include high hardness, brittleness, and no ductility.

- What do bauxite and corundum have in common?

- Answer. They are both minerals of alumina.

- What is clay, used in making ceramic products?

- Answer. Clay most commonly consists of hydrous aluminum silicate; for example, kaolinite (Al2(Si2O5)(OH)4) is a common clay.

- What are some of the principal applications of cemented carbides, such as WC-Co?

- Answer. Important applications of WC-Co include cutting tool inserts, drawing dies, rock drilling bits, dies for powder metallurgy, and other applications where hardness is a critical factor.

- What is one of the important applications of titanium nitride, as mentioned in the text?

- Answer. As a thin coating on cutting tools to prolong tool life.

- What is the primary mineral in glass products?

- Answer. Silica, or silicon dioxide (SiO2).

- What does the term devitrification mean?

- Answer. Devitrification is the transformation from the glassy state into a polycrystalline state.

- What is a polymer?

- Answer. A polymer is a compound comprised of long-chain molecules that consist of repeating units, called mers, connected end to end.

- What are the three basic categories of polymers?

- Answer. The categories are (1) thermoplastics, (2) thermosetting polymers, and (3) elastomers.

- How do the properties of polymers compare with those of metals?

- Answer. In general, polymers have lower strength, hardness, stiffness, density, and temperature resistance compared to metals.  In addition, polymers have low electrical and thermal conductivity.

- What is cross-linking in a polymer, and what is its significance?

- Answer. Cross-linking is the formation of connections between the long-chain molecules in a polymer.  It causes the polymer structure to be permanently altered.  If the amount of cross-linking is low, the polymer is transformed into an elastomer; if cross-linking is significant, the polymer is transformed into a thermoset.

- The nylons are members of which polymer group?

- Answer. Polyamides.

- What is the chemical formula of ethylene, the monomer for polyethylene?

- Answer. The chemical formula of ethylene is C2H4.

- How do the properties of thermosetting polymers differ from those of thermoplastics?

- Answer. Thermosets are more rigid, brittle, capable of higher service temperatures, and cannot be remelted.

- Cross-linking (curing) of thermosetting plastics is accomplished by one of three ways.  Name the three ways.

- Answer. The three ways are (1) temperature-activated systems, in which elevated temperatures accomplish curing; (2) catalyst-activated systems, in which small amounts of a catalyst cause cross-linking; and (3) mixing-activated systems, in which two reactive components are mixed and curing occurs by their chemical reaction.

- Elastomers and thermosetting polymers are both cross-linked. Why are their properties so different?

- Answer. Elastomers are lightly cross-linked, whereas thermosets are highly cross-linked.  Light cross-linking allows extensibility; a highly cross-linked structure makes the polymer rigid.

- What is the primary polymer ingredient in natural rubber?

- Answer. The primary polymer ingredient in natural rubber is polyisoprene whose mer has the chemical formula C5H8.

- How do thermoplastic elastomers differ from conventional rubbers?

Answer. TPEs are different in two basic ways: (1) they exhibit thermoplastic properties, and (2) their extensibility derives from physical connections between different phases in the polymer rather than cross-linking.

- What is a composite material?

- Answer. A composite material is a material system consisting of two or more distinct phases whose combination results in properties that differ from those of its constituents.

- Identify some of the characteristic properties of composite materials.

- Answer. Typical properties include (1) high strength-to-weight and stiffness-to-weight ratios; (2) good fatigue properties and toughness; (3) anisotropic properties in many cases; and (4) other properties and features that are difficult or impossible to obtain with metals, ceramics, or polymers alone.

- What does the term anisotropic mean?

- Answer. Anisotropic means that the properties of a material vary depending on the direction in which they are measured.

- Name the three basic categories of composite materials.

- Answer. Metal matrix composites (MMCs), ceramic matrix composites (CMCs), and polymer matrix composites (PMCs).

- What are the common forms of the reinforcing phase in composite materials?

- Answer. The forms are: (1) fibers, (2) particles and flakes, and (3) an infiltrated phase in skeletal structures.

- What is a cermet?

- Answer. A cermet is a composite material consisting of a ceramic and a metal.  In the text, it is defined as a composite consisting of ceramic grains imbedded in a metallic matrix.

- Cemented carbides are what class of composites?

- Answer. A cemented carbide is a cermet; although the cemented carbide industry does not generally think of cemented carbides as cermets, they fit within the definition.

- What are some of the weaknesses of ceramics that might be corrected in fiber-reinforced ceramic matrix composites?

- Answer. Weaknesses of ceramics include low tensile strength, poor toughness, and susceptibility to thermal cracking.

- What is the most common fiber material in fiber-reinforced plastics?

- Answer. E-glass.

- Identify some of the important properties of fiber-reinforced plastic composite materials.

- Answer. Properties include high strength-to-weight ratio, high modulus-to-weight ratio, low density, good fatigue strength, good corrosion resistance, and low thermal expansion for many FRPs.

- Name some of the important applications of FRPs.

- Answer. FRPs are used as components in modern aircraft, automobile and truck body panels, tennis rackets, boat hulls, and a variety of other sports and recreational items.