25 MECHANICAL ASSEMBLY

# Review Questions

- How does mechanical assembly differ from the other methods of assembly discussed in previous chapters (e.g., welding, brazing, etc.)?

- Answer. Mechanical assembly uses a mechanical fastening method for joining two (or more) parts, whereas welding, brazing, soldering, and adhesive bonding use heat and/or pressure, sometimes combined with a filler material to permanently join parts. Also, many of the mechanical fastening methods allow for disassembly - not possible with welding and brazing.

- What are some of the reasons why assemblies must be sometimes disassembled?

- Answer. For maintenance and repair service, to replace worn-out components, and to make adjustments.

- What is the technical difference between a screw and a bolt?

- Answer. Both are externally threaded fasteners.  A screw is generally assembled into a blind threaded hole, whereas a bolt is assembled using a nut.

- What is a stud (in the context of threaded fasteners)?

- Answer. A stud is an externally threaded fastener that does not have the usual head possessed by a bolt.

- What is torque-turn tightening?

- Answer. Torque-turn tightening involves the tightening of the threaded fastener to a certain low torque level, and then advancing the fastener by a specified additional amount of turn (e.g., a quarter turn).

- Define proof strength as the term applies in threaded fasteners.

- Answer. Proof strength can be defined as the maximum tensile stress that an externally threaded fastener can sustain without permanent deformation.

- What are the three ways in which a threaded fastener can fail during tightening?

- Answer. (1) Stripping of the bolt or screw threads, (2) stripping of the internal fastener threads, and (3) excessive tensile load on the cross-sectional area of the bolt or screw.

- What is a rivet?

- Answer. A rivet is an unthreaded headed pin used to join two parts by inserting the pin through holes in the parts and deforming the unheaded portion over the opposite side.

- What is the difference between a shrink fit and expansion fit in assembly?

- Answer. In a shrink fit, the outside part is expanded by heating to fit over the mating component.  Then cooling causes an interference fit with the component.  In an expansion fit, the internal part is cooled so that it can be readily inserted into the mating component.  Then, upon warming to room temperature, it expands to cause an interference fit with its mating part.

- What are the advantages of snap fitting?

- Answer. Advantages of snap fitting include (1) the method is fast, (2) no tooling is required, and (3) the parts can be designed with self-aligning features for ease of mating.

- What is the difference between industrial stitching and stapling?

- Answer. In stitching the U-shaped fasteners are formed during the assembly process.  In stapling, the fasteners are preformed.

- What are integral fasteners?

- Answer. Integral fasteners make use of a forming operation on one of the parts to be joined to interlock the components and create a mechanically fastened joint.

- Identify some of the general principles and guidelines for design for assembly.

- Answer. Some of the general principles and guidelines in design for assembly include the following: (1) Use the fewest number of parts possible to reduce assembly required. (2) Reduce the number of threaded fasteners; instead use snap fits, retaining rings, integral fasteners, and similar fastening mechanisms that can be accomplished more rapidly. Use threaded fasteners only where justified, e.g., where disassembly or adjustment is required. (3) Standardize fasteners in order to reduce the number of sizes and styles in the product. (4) Design parts to be symmetrical and minimize asymmetric features. (5) Avoid parts that tangle.

- Identify some of the general principles and guidelines that apply specifically to automated assembly.

Answer. Some of the principles and guidelines that apply specifically to automated assembly include the following: (1) Use modularity in product design. Each module to be produced on a single assembly system should have a maximum of 12 or 13 parts and should be designed around a base part to which other components are added. (2) Reduce the need for multiple components to be handled at once. (3) Limit the required directions of access. The ideal is for all components to be added vertically from above. (4) Use only high quality components. Poor quality components cause jams in feeding and assembly mechanisms. (5) Use snap fit assembly to eliminate the need for threaded fasteners.

# Problems

- A 5-mm-diameter bolt is to be tightened to produce a preload = 250 N. If the torque coefficient = 0.23, determine the torque that should be applied.

- Solution: T = CtDF = 0.23(5.0)(250) = 287.5 N-mm = 0.2875 N-m

- An alloy steel Metric 10x1.5 screw (10 mm diameter, pitch p = 1.5 mm) is to be turned into a threaded hole and tightened to one/half of its proof strength. According to Table 25.1, the proof strength = 830 MPa. Determine the maximum torque that should be used if the torque coefficient = 0.18.

- Solution: As = 0.25(10 - 0.9382 x 1.5)2 = 57.99 mm2

-  = 0.5 of 830 MPa = 415 MPa = 415 N/mm2

- F = As = 415(57.99) = 24,066 N

- T = CtDF = 0.18(10)(24,066) = 43,319 N-mm = 43.32 N-m

- A Metric 16x2 bolt (16 mm diameter, 2 mm pitch) is subjected to a torque of 15 N-m during tightening. If the torque coefficient is 0.24, determine the tensile stress on the bolt.

- Solution: T = 15 N-m = 15,000 N-mm

- F = T/ CtD = 15,000/(0.24 x 16) = 3906 N

- As = 0.25(16 - 0.9382 x 2)2 = 156.7 mm2

-  = 3906/156.7 = 24.9 N/mm2 = 24.9 MPa

- A 1/2-13 screw is to be preloaded to a tension force = 1000 lb. Torque coefficient = 0.22. Determine the torque that should be used to tighten the bolt.

- Solution: T = CtDF = 0.22(0.50)(1000) = 110 in-lb

- A torque wrench is used on a 3/4-10 bolt in an automobile final assembly plant. A torque of 70 ft-lb is generated by the wrench. If the torque coefficient = 0.17, determine the tensile stress in the bolt.

- Solution: T = 70 ft-lb = 840 in-lb

- F = T/CtD = 840/(0.17 x 0.75) = 6588 lb.

- As = 0.25(0.75 - 0.9743/10)2 = 0.334 in2

-  = 6588/0.334 = 19,697 lb/in2

- The designer has specified that a 3/8-16 low-carbon bolt (3/8 in nominal diameter, 16 threads/in) in a certain application should be stressed to its proof stress of 33,000 lb/in2 (from Table 25.1). Determine the maximum torque that should be used if the torque coefficient = 0.25.

- Solution: As = 0.25(0.375 - 0.9743/16)2 = 0.0775 in2

- F = As = 33,000(0.0775) = 2557.5 lb

- T = CtDF = 0.25(0.375)(2557.5) = 240 in-lb

- A dowel pin made of steel (elastic modulus = 209,000 MPa) is to be press fitted into a steel collar. The pin has a nominal diameter of 16.0 mm, and the collar has an outside diameter of 27.0 mm. (a) Compute the radial pressure and the maximum effective stress if the interference between the shaft OD and the collar ID is 0.03 mm. (b) Determine the effect of increasing the outside diameter of the collar to 39.0 mm on the radial pressure and the maximum effective stress.

- Solution: (a) pf = Ei(Dc2 - Dp2)/DpDc2 = 209,000(0.03)(272 - 162)/(16 x 272) = 254 MPa

- Max e = 2pfDc2/(Dc2 - Dp2) = 2(254.3)(272)/(272 - 162) = 784 MPa

- (b) When Dc = 39 mm, pf = 209,000(0.03)(392 - 162)/(16 x 392) = 326 MPa

- Max e  = 2(325.9)(392)/(392 - 162) = 784 MPa

- A gear made of aluminum (modulus of elasticity = 69,000 MPa) is press fitted onto an aluminum shaft. The gear has a diameter of 55 mm at the base of its teeth. The nominal internal diameter of the gear = 30 mm and the interference = 0.10 mm. Compute: (a) the radial pressure between the shaft and the gear, and (b) the maximum effective stress in the gear at its inside diameter.

- Solution: (a) pf = Ei(Dc2 - Dp2)/DpDc2 = 69,000(0.10)(552 - 302)/(30 x 552) = 161.5 MPa

- (b) Max e = 2pfDc2/(Dc2 - Dp2) = 2(161.5)(552)/ (552 - 302) = 460 MPa

- A steel collar is press fitted onto a steel shaft. The modulus of elasticity of steel is 30 x 106 lb/in2. The collar has an internal diameter of 2.498 in and the shaft has an outside diameter = 2.500 in. The outside diameter of the collar is 4.000 in. Determine the radial (interference) pressure on the assembly, and (b) the maximum effective stress in the collar at its inside diameter.

- Solution: (a) i = 2.500 – 2.498 = 0.002 in

- pf = Ei(Dc2 - Dp2)/DpDc2 = 30 x 106(0.002)(4.0002 -2.5002)/(2.500 x 4.0002) = 14,625 lb/in2

- (b) Max e = 2pfDc2/(Dc2 - Dp2) = 2(14,625)(4.0002)/ (4.0002 – 2.5002) = 48,000 lb/in2

- The yield strength of a certain metal = 50,000 lb/in2 and its modulus of elasticity = 22 x 106 lb/in2. It is to be used for the outer ring of a press-fit assembly with a mating shaft made of the same metal. The nominal inside diameter of the ring is 1.000 in and its outside diameter = 2.500 in. Using a safety factor = 2.0, determine the maximum interference that should be used with this assembly.

- Solution: Max e  Y/SF , use Max e = Y/SF = 50,000/2.0 = 25,000 lb/in2 	Eq. (25.9)

- Max e = 2pfDc2/(Dc2 - Dp2) = 25,000 lb/in2					Eq. (25.6)

- Rearranging, pf = e(Dc2 - Dp2)/2Dc2 = 25,000(2.52 - 1.02)/(2 x 2.52) = 10,500 lb/in2

- pf = Ei(Dc2 -Dp2)/DpDc2 								Eq. (25.5)

- Rearranging, i = pf DpDc2/E(Dc2 - Dp2)

- i = 10,500(1.0)(2.52)/(22 x 106 (2.52 - 1.02)) = 0.00057 in

- A shaft made of aluminum is 40.0 mm in diameter at room temperature (21C). Its coefficient of thermal expansion = 24.8 x 10-6 mm/mm per C. If it must be reduced in size by 0.20 mm in order to be expansion fitted into a hole, determine the temperature to which the shaft must be cooled.

- Solution: (D2 - D1) = -0.20 = 24.8 x 10-6(40)(T2 - 21)

- T2 - 21 = -0.20/(24.8 x 10-6 x 40) = -201.6

- T2 = -201.6 + 21 = -180.6C

- A steel ring has an inside diameter = 30 mm and an outside diameter = 50 mm at room temperature (21C). If the coefficient of thermal expansion of steel = 12.1 x 10-6 mm/mm per C, determine the inside diameter of the ring when heated to 500C.

- Solution: D2 - D1 = D2 - 30 = 12.1 x 10-6(30)(500 - 21)

- D2 = 30 + 0.174 = 30.174 mm.

- A steel collar is to be heated from room temperature (70F) to 700F. Its inside diameter = 1.000 in, and its outside diameter = 1.625 in. If the coefficient of thermal expansion of the steel is = 6.7 x 10-6 in/in per F, determine the increase in the inside diameter of the collar.

- Solution: (D2 - D1) = D1(T2 - T1) = 6.7 x 10-6(1.0)(700 - 70) = 4221 x 10-6 = 0.0042 in

- A steel collar whose outside diameter = 3.000 in at room temperature is to be shrink fitted onto a steel shaft by heating it to an elevated temperature while the shaft remains at room temperature. The shaft diameter = 1.500 in. For ease of assembly when the collar is heated to a temperature of 1000F, the clearance between the shaft and the collar is to be 0.007 in. Determine (a) the initial inside diameter of the collar at room temperature so that this clearance is satisfied, (b) the radial pressure and (c) maximum effective stress on the resulting interference fit at room temperature (70F). For steel, the elastic modulus = 30,000,000 lb/in2 and coefficient of thermal expansion = 6.7 x 10-6 in/in per F.

- Solution: (a) If the clearance = 0.007 in, then the inside diameter of the collar must be

- D2 = Dp + 0.007 = 1.500 + 0.007.

- 1.507 - D1 = 6.7 x 10-6 D1(1000 - 70)

- 1.507 - D1 = 0.00623 D1

- 1.507 = D1 + 0.00623 D1 = 1.00623 D1

- D1 = 1.507/1.00623 = 1.4977 in

- (b) Interference i = 1.500 - 1.4977 = 0.00233 in

- pf = 30 x 106 (0.00233)(3.02 - 1.52)/(1.5 x 3.02) = 34,950 lb/in2

- (c) Max e  = 2(34,950)(3.02)/ (3.02 - 1.52) = 93,200 lb/in2