4 DIMENSIONS, TOLERANCES AND SURFACES

Review Questions

- What is a tolerance?

- Answer. A tolerance is defined as the total amount by which a specified dimension is permitted to vary.

- What is the difference between a bilateral tolerance and a unilateral tolerance?

- Answer. A bilateral tolerance allows variation in both positive and negative directions from the nominal dimension, whereas a unilateral tolerance allows the variation from the nominal dimension to be either positive or negative, but not both.

- What is accuracy in measurement?

- Answer. Accuracy is the degree to which the measured value agrees with the true value of the quantity of interest.  It is a measurement procedure that is absent of systematic errors.

- What is precision in measurement?

- Answer. Precision in measurement is the degree to which random errors are minimized.

- What are some of the reasons why surfaces are important?

- Answer. The reasons why surfaces are important include: aesthetics, safety, friction and wear, effect of surface on mechanical and physical properties, mating of components in assembly, and electrical contacts.

- Define nominal surface.

- Answer. The nominal surface is the ideal part surface represented on an engineering drawing. It is assumed perfectly smooth; perfectly flat if referring to a planar surface; perfectly round if referring to a round surface, etc.

- Define surface texture.

- Answer. Surface texture is the random and repetitive deviations from the nominal surface, including roughness, waviness, lay, and flaws.

- How is surface texture distinguished from surface integrity?

- Answer. Surface texture refers only to the surface geometry; surface integrity includes not only surface but the subsurface layer beneath the surface and the changes in it.

- Within the scope of surface texture, how is roughness distinguished from waviness?

- Answer. Roughness consists of the finely-spaced deviations from the nominal surface, while waviness refers to the deviations of larger spacing.  Roughness deviations lie within waviness deviations.

- Surface roughness is a measurable aspect of surface texture; what does surface roughness mean?

- Answer. Surface roughness is defined as the average value of the vertical deviations from the nominal surface over a specified surface length.

- Indicate some of the limitations of using surface roughness as a measure of surface texture.

- Answer. Surface roughness provides only a single measure of surface texture. Among its limitations are: (1) it varies depending on direction; (2) it does not indicate lay; (3) its value depends on the roughness width cutoff used to measure the average.

- What causes the various types of changes that occur in the altered layer just beneath the surface?

- Answer. Energy input to the surface resulting from the manufacturing process used to generate the surface. The energy forms can be any of several types, including mechanical, thermal, chemical, and electrical.

- Name some manufacturing processes that produce very poor surface finishes.

- Answer. Processes that produce poor surfaces include sand casting, hot rolling, sawing, and thermal cutting (e.g., flame cutting).

- Name some manufacturing processes that produce very good or excellent surface finishes.

Answer. Processes that produced very good and excellent surfaces include honing, lapping, polishing, and superfinishing.