30 QUALITY CONTROL AND INSPECTION

# Review Questions

- What are the two principal aspects of product quality?

- Answer. The two quality aspects are (1) product features and (2) freedom from deficiencies.

- How is a process operating in statistical control distinguished from one that is not?

- Answer. The process in statistical control is characterized by only random variations.  A process that is out of control exhibits additional variation that is not normal, called assignable variation.  This indicates that something is wrong with the process.

- Define process capability.

- Answer. Process capability is the limits of the random variations of the process when it is in statistical control. The limits are defined as the process mean  3 standard deviations.

- What are the natural tolerance limits?

- Answer. The natural tolerance limits are when the tolerance on a part is set equal to the process capability; that is 3 of the process mean.

- What is the difference between control charts for variables and control charts for attributes?

- Answer. In control charts for variables, measurements of the characteristic of interest are made.  In control charts for attributes, the characteristic of interest is identified as being acceptable of not acceptable.

- Identify the two types of control charts for variables.

- Answer. The two charts are (1)x chart for sample means and (2) R chart for ranges.

- What are the two basic types of control charts for attributes?

- Answer. The two charts are (1) p chart for proportion of defects in a sample, and (2) c chart for count of defects in a sample.

- When interpreting a control chart, what does one look for to identify problems?

Answer. Problems are indicated by the following: (1) x or R lie outside their respective LCL or UCL limits; (2) trends or cyclical patterns in the data; (3) sudden changes in average; and (4) points consistently near the upper or lower limits.

- What are the three main goals in total quality management (TQM)?

Answer. The three main goals in TQM are (1) achieving customer satisfaction, (2) encouraging the involvement of the entire workforce, and (3) continuous improvement.

- What is the difference between external customers and internal customers in TQM?

Answer. External customers are those who purchase the company’s products and services. Internal customers are inside the company, such as the company’s final assembly department which is the customer of the parts production departments.

  - At what company was the Six Sigma quality program first used?

Answer. Motorola Corporation.

- Why is the normal statistical table used in a Six Sigma program different from the standard normal tables found in textbooks on probability and statistics?

- Answer. The normal statistical tables used in a Six Sigma program differ from the standard normal tables in the following two ways: (1) the Six Sigma table includes only one tail of the normal distribution and (2) the Six Sigma table is shifted by 1.5, so that 6 in the Six Sigma table is the same as 4.5  in the standard normal tables.

- A Six Sigma program uses three measures of defects per million (DPM) to assess the performance of a given process. Name the three measures of DPM.

- Answer. The three measures of DPM are (1) defects per million opportunities, (2) defects per million units, and (3) defective units per million units.

- Automated inspection can be integrated with the manufacturing process to accomplish certain actions. What are these possible actions?

- Answer. Possible actions discussed in text are (1) parts sortation and (2) feedback of data to adjust the process.

- Give an example of a noncontact inspection technique.

- Answer. Non-contact inspection techniques include machine vision and electrical field techniques.

- What is a coordinate measuring machine?

- Answer. A CMM is an automated measuring machine consisting of a contact probe and a means to position the probe in three dimensions relative to workpart features and surfaces; when the probe contacts the part, the x-y-z coordinates are recorded.

- What is a binary vision system?

- Answer. In a binary vision system, the light intensity of each pixel is reduced to either of two values (black or white, 0 or 1).

- Name some of the nonoptical noncontact sensor technologies available for inspection.

Answer. The technologies include electrical fields (capacitance, inductance), radiation (X-ray), and ultrasonic techniques (high frequency sound).

# Problems

- An automatic turning process is set up to produce parts with a mean diameter = 6.255 cm. The process is in statistical control and the output is normally distributed with a standard deviation = 0.004 cm. Determine the process capability.

- Solution: Process capability PC =   3 = 6.255  3(0.004) = 6.255  0.012 cm

- The upper and lower limits of the process capability range are: 6.243 to 6.267 cm

- A sheet-metal bending operation produces bent parts with an included angle = 92.1. The process is in statistical control and the values of included angle are normally distributed with a standard deviation = 0.23. The design specification on the angle = 90  2. Determine the process capability.

- Solution: PC = 92.1  3(0.23) = 92.1  0.69.

- The upper and lower limits of the process capability range are: 91.41 to 92.79. Note that the design tolerance specifies an acceptable range between 88 and 92, so more than half of the output is outside of the tolerance range.

- A plastic extrusion process produces round tubular extrudate with a mean outside diameter = 28.6 mm. The process is in statistical control and the output is normally distributed with standard deviation = 0.53 mm. Determine the process capability.

- Solution: Process capability PC =   3 = 28.6  3(0.53) = 28.6  1.59 mm

- The upper and lower limits of the process capability range are: 27.01 to 30.19 mm.

- In 12 samples of size n = 7, the average value of the sample means is x = 6.860 cm for the dimension of interest, and the mean of the ranges of the samples is R = 0.027 cm. Determine (a) lower and upper control limits for the x chart and (b) lower and upper control limits for the R chart.

- Solution: (a) x chart: x = 6.860 cm = CL

- LCL = x - A2R= 6.860 - 0.419(0.027) = 6.8487 cm

- UCL = x + A2R= 6.860 + 0.419(0.027) = 6.8713 cm

- (b) R chart: R = 0.027 = CL

- LCL = D3R= 0.076(0.027) = 0.0205 cm

- UCL = D4R= 1.924(0.027) = 0.0519 cm

- In nine samples of size n = 10, the grand mean of the samples is x = 100 for the characteristic of interest, and the mean of the ranges of the samples is R = 8.5. Determine (a) lower and upper control limits for the x chart and (b) lower and upper control limits for the R chart.

- Solution: (a) x chart: x = 100 = CL

- LCL = x - A2R= 100 - 0.308(8.5) = 102.618

- UCL = x + A2R= 100 + 0.308(8.5) = 97.382

- (b) R chart: R = 8.5 = CL

- LCL = D3R= 0.223(8.5) = 1.8955

- UCL = D4R= 1.777(8.5) = 15.1045

- Ten samples of size n = 8 have been collected from a process in statistical control, and the dimension of interest has been measured for each part. The calculated values of x for each sample are (mm) 9.22, 9.15, 9.20, 9.28, 9.19, 9.12, 9.20, 9.24, 9.17, and 9.23. The values of R are (mm) 0.24, 0.17, 0.30, 0.26, 0.26, 0.19, 0.21, 0.32, 0.21, and 0.23, respectively. (a) Determine the values of the center, LCL, and UCL for the x and R charts. (b) Construct the control charts and plot the sample data on the charts.

Solution: x= x/m = x/10

= (9.22 + 9.15 + 9.20 + 9.28 + 9.19 + 9.12 + 9.20 + 9.24 + 9.17 + 9.23)/10 = 9.20

R=  R/10 = (0.24 + 0.17 + 0.30 + 0.26 + 0.27 + 0.19 + 0.21 + 0.32 + 0.21 + 0.23)/10 = 0.24

(a) xchart: x= 9.20 mm = CL

LCL = x - A2R= 9.20 - 0.373(0.24) = 9.1105 mm.

UCL = x + A2R= 9.20 + 0.373(0.24) = 9.2895 mm.

R chart: R = 0.024 = CL

LCL = D3R= 0.136(0.24) = 0.0326 mm.

UCL = D4R= 2.114(0.0133) = 0.4474 mm.

(b) Student exercise.

- Seven samples of 5 parts each have been collected from an extrusion process that is in statistical control, and the diameter of the extrudate has been measured for each part. The calculated values of x for each sample are (in) 1.002, 0.999, 0.995, 1.004, 0.996, 0.998, and 1.006. The values of R are (in) 0.010, 0.011, 0.014, 0.020, 0.008, 0.013, and 0.017, respectively. (a) Determine the values of the center, LCL, and UCL for x and R charts. (b) Construct the control charts and plot the sample data on the charts.

Solution: x = x/7 = (1.002 + 0.999 + 0.995 + 1.004 + 0.996 + 0.998 + 1.006)/7 = 1.000

R	=  R/7 = (0.010 + 0.011 + 0.014 + 0.020 + 0.008 + 0.013 + 0.017)/7 = 0.0133

(a) xchart: x= 1.000 in = CL

LCL = x - A2R= 1.000 - 0.577(0.0133) = 0.9923 in

UCL = x + A2R= 1.000 + 0.577(0.0133) = 1.0077 in

R chart: R = 0.0133 = CL

LCL = D3R= 0(0.0133) = 0

UCL = D4R= 2.114(0.0133) = 0.0281 in

(b) Student exercise.

- A p chart is to be constructed. Six samples of 25 parts each have been collected, and the average number of defects per sample was 2.75. Determine the center, LCL and UCL for the p chart.

- Solution: p= 2.75/25 = 0.11 = CL

- LCL = p- 3p(1−p)n =  0.11 - 30.11(0.89)25= 0.11 - 3(0.0626) = -0.078  0

- UCL = p+ 3p(1−p)n =  0.11 + 30.11(0.89)25= 0.11 + 3(0.0626) = 0.298

- Ten samples of equal size are taken to prepare a p chart. The total number of parts in these ten samples was 900 and the total number of defects counted was 117. Determine the center, LCL and UCL for the p chart.

- Solution: d= 117/10 = 11.7.

- p= 11.7/90 = 0.13 = CL

- LCL = p- 3 = 0.13 - 30.13(0.87)90= 0.13 - 3(0.03545) = 0.024

- UCL = p+ 3 = 0.11 + 30.13(0.87)90= 0.11 + 3(0.03545) = 0.236

- The yield of good chips during a certain step in silicon processing of integrated circuits averages 91%. The number of chips per wafer is 200. Determine the center, LCL, and UCL for the p chart that might be used for this process.

- Solution: Use p= 1 - 0.91 = 0.09 = CL

- LCL = p- 3 = 0.09 - 30.09(0.91)90= 0.09 - 3(0.0202) = 0.0293

- UCL = p+ 3 = 0.11 + 30.09(0.91)90= 0.09 + 3(0.0202) = 0.1507

- Twelve cars were inspected after final assembly. The number of defects found ranged between 87 and 139 defect per car with an average of 116. Determine the center and upper and lower control limits for the c chart that might be used in this situation.

Solution: CL = 116

LCL = c- 3c= 116 - 3116= 83.7  83

UCL = c+ 3c= 116 + 3116= 148.3  148

- A foundry that casts turbine blades inspects for eight features that are considered critical-to-quality. During the previous month, 1,236 castings were produced. During inspection, 47 defects among the eight features were found, and 29 castings had one or more defects. Determine DPMO, DPM, and DUPM in a Six Sigma program for these data and convert each to its corresponding sigma level.

Solution: Summarizing the data, Nu = 1236, No = 8, Nd = 47, and Ndu = 29. Thus,

The corresponding sigma level is about 4.1 from Table 30.2.

The corresponding sigma level is about 3.3.

The corresponding sigma level is about 3.4.

- In the previous problem, if the foundry desired to improve its quality performance to the 5.0 sigma level in all three measures of DPM, how many defects and defective units would they produce in an annual production quantity of 15,000 castings? Assume the same eight features are used to assess quality.

Solution: Summarizing the data, Nu = 15,000, No = 8, and Nd  and Ndu are unknown. To achieve a 5.0 sigma level, they would produce 233 dpm. Thus, for defects per million opportunities,

Rearranging, Nd = 233(15,000)(8)/1,000,000 = 27.96, rounded up to 28 defects in 15,000 total castings.

Similarly, if the 5.0 sigma criterion were used for defects per million units,

Rearranging, Nd = 233(15,000)/1,000,000 = 3.5 defects in 15,000 total castings.

Finally, if the 5.0 sigma criterion were used for defective units per million units,

Ndu = 233(15,000)/1,000,000 = 3.5 defective units in 15,000 total castings.

- The inspection department in an automobile final assembly plant inspects cars coming off the production line against 55 quality features considered important to customer satisfaction. The department counts the number of defects found per 100 cars, which is the same type of metric used by a national consumer advocate agency. During a one-month period, a total of 16,582 cars rolled off the assembly line. These cars included a total of 6045 defects of the 55 features, which translates to 36.5 defects per 100 cars. In addition, a total of 1955 cars had one or more of the defects during this month. Determine DPMO, DPM, and DUPM in a Six Sigma program for these data and convert each to its corresponding sigma level.

Solution: Although the inspection department uses number of defects per 100 cars, a Six Sigma program uses defects per million as its metric. Summarizing the data, Nu = 16,582, No = 55, Nd = 6045, and Ndu = 1955. Thus,

The corresponding sigma level is about 4.0 from Table 30.2.

The corresponding sigma level is about 1.8.

The corresponding sigma level is about 2.7.