11 PROCESSING OF CERAMICS AND CERMETS

# Review Questions

- What is the difference between the traditional ceramics and the new ceramics, as far as raw materials are concerned?

- Answer. The traditional ceramics are based on hydrous aluminum silicates (clay), whereas the new ceramics are based on simpler compounds such as oxides, nitrides, and carbides.

- List the basic steps in the traditional ceramics processing sequence.

- Answer. The sequence is (1) preparation of raw materials, (2) shaping, (3) drying, and (4) firing.

- What is the technical difference between crushing and grinding in the preparation of traditional ceramic raw materials?

- Answer. Crushing is performed to reduce large lumps of mineral to smaller size. Grinding is a secondary process which further reduces the particle size to fine powder.

- Describe the slip casting process in traditional ceramics processing.

- Answer. In slip casting, a slurry of clay is poured into a plaster of Paris mold, whereupon water is absorbed from the slurry into the plaster to form a clay layer against the mold wall. The remaining slurry is usually poured out to leave a hollow part.

- List and briefly describe some of the plastic forming methods used to shape traditional ceramic products.

- Answer. The plastic forming methods include (1) hand modeling, molding, and throwing; (2) jiggering, which is a mechanized extension of hand throwing used to manufacture bowls and plates; (3) plastic pressing, in which a clay slug is pressed in a mold; and (4) extrusion, in which the clay is compressed through a die opening to make long pieces of uniform cross section.

- What is the process of jiggering?

- Answer. Jiggering is a clay forming process that uses a convex mold on a potters wheel. The clay is first pressed into rough shape and then rotated and formed with a jigger tool to final shape. It is suited to the manufacture of flatware (e.g., dinner plates).

- What is the difference between dry pressing and semi-dry pressing of traditional ceramic parts?

- Answer. The difference is in the starting clay. For semi-dry pressing, the clay has a typical water content of 10% to 15%.  For dry pressing, the water content is usually less than 5%.  Dry clay has virtually no plasticity, and so this imposes certain limitations on part geometry in dry pressing.

- What happens to a ceramic material when it is sintered?

- Answer. Sintering of green ceramics (or powdered metals) causes bonding between the ceramic grains, which is accompanied by densification and reduction of porosity.

- What is the name given to the furnace used to fire ceramic ware?

- Answer. Kiln.

- Why is the drying step, so important in the processing of traditional ceramics, usually not required in processing of new ceramics?

- Answer. Because water is usually not one of the ingredients in the new ceramics during forming.  Drying is only needed when the green piece contains water.

- What is the freeze drying process used to make certain new ceramic powders?

- Answer. In freeze drying, salts are dissolved in water and sprayed into small droplets that are immediately frozen; the water is then removed from the droplets in a vacuum chamber, and the freeze-dried salt is decomposed by heating to form the ceramic powders.

- Describe the doctor-blade process.

- Answer. In the doctor-blade process, a ceramic slurry is flowed onto a moving film which passes under a wiper blade, so that the resulting ceramic is in the form of a thin green sheet which is dried and reeled onto a spool for subsequent shaping and sintering.

- What are some design recommendations for ceramic parts?

Answer. The guidelines include the following: (1) subject ceramic parts to compressive stresses, not tensile stresses; (2) ceramics are brittle, so avoid impact loading; (3) part geometries should be simple; (4) use large radii on inside and outside corners; (5) take into account shrinkage; and (6) no screw threads.