29 SURVEY OF AUTOMATION AND MANUFACTURING SYSTEMS

###### Review Questions

- Define the term manufacturing system.

- Answer. The text defines manufacturing system as a collection of integrated equipment and human resources that performs processing and/or assembly operations on a starting work material, part, or set of parts. The integrated equipment consists of one or more production machines, material handling and positioning devices, and computer systems.

  - Identify and briefly describe the three basic components of a numerical control system.

- Answer. The three basic components are: (1) part program, (2) machine control unit, and (3) processing equipment. The part program is the detailed set of commands to be followed by the processing equipment. The machine control unit in modern NC technology is a microcomputer that stores the program and executes it by converting each command into actions by the processing equipment, one command at a time. The processing equipment accomplishes the sequence of processing steps to transform the starting workpart into a completed part.

- What is the difference between point-to-point and continuous path in a motion control system?

- Answer. In point-to-point, the motion is from one location in space to the next with no regard for the path taken between starting and final locations. In continuous path, the trajectory of the movement is controlled.

- What is the difference between absolute positioning and incremental positioning?

- Answer. In absolute positioning, the locations are defined relative to the origin of the axis system.  In incremental positioning, each succeeding location is defined relative to the previous location.

- What is the difference between an open-loop positioning system and a closed-loop positioning system?

- Answer. In a closed loop system, measurements of the output (position) are fed back to verify that it corresponds to the desired input value.  In an open loop system, there is no feedback of the output value.

- Under what circumstances is a closed-loop positioning system preferable to an open-loop system?

- Answer. When there is a significant reaction force resisting the motion of the positioning system, a closed loop system is preferred.

- Explain the operation of an optical encoder.

- Answer. An optical encoder is a sensor for measuring angular position and rotational velocity. It consists of a light source, a photodetector, and a disk containing a series of slots through which the light source can shine to energize the photodetector. The disk is connected, either directly or through a gear train, to a rotating shaft whose angular position and velocity are to be measured. As the shaft rotates, the slots cause the light source to be seen by the photocell as a series of flashes, which are converted into an equivalent series of electrical pulses. By counting the pulses and computing the frequency of the pulse train, angular position and rotational speed can be determined.

- What is manual data input in NC part programming?

- Answer. Manual data input refers to a method of programming in which the machine tool operator accomplishes the programming of the NC machine using a menu-driven procedure.  Programming is simplified to minimize the amount of training required by the operator.

- Identify some of the non-machine tool applications of numerical control.

- Answer. The applications include (1) tape-laying machines and filament-winding machines for composites; (2) welding machines, both arc welding and resistance welding; (3) component-insertion machines in electronics assembly; (4) drafting machines; and (5) coordinate measuring machines for inspection.

- What are some of the benefits usually cited for NC compared to using manual alternative methods?

- Answer. Advantages of NC include (1) reduced non-productive time, (2) lower manufacturing lead times, (3) simpler fixtures, (4) greater flexibility, (5) improved accuracy, and (6) reduced human error.

- Define group technology.

- Answer. GT is a general approach in which similarities among parts are identified and exploited in design and manufacturing.

- What is a part family?

- Answer. A part family is a collection of parts that share similar design and/or manufacturing attributes.

- Define cellular manufacturing.

- Answer. Cellular manufacturing involves the production of part families using groups of machines (generally manually operated) to produce a certain part family or a limited set of part families.

- What is the composite part concept in group technology?

- Answer. In GT, a composite part is a hypothetical part that includes all of the design and/or manufacturing attributes of a given part family.  The concept is useful in designing cells to produce the part family.

- What is a flexible manufacturing system?

- Answer. A flexible manufacturing system (FMS) is an automated group technology cell consisting of processing stations interconnected by an automated handling system and controlled by a computer.

- What are the criteria that should be satisfied to make an automated manufacturing system flexible?

- Answer. The flexibility criteria for an FMS are (1) the system must process different part styles in non-batch mode; (2) the system must be able to accept changes in the production schedule, (3) the system must deal gracefully with equipment breakdowns, and (4) the system must be able to accommodate new part style introductions.

- Name some of the FMS software and control functions.

- Answer. FMS software and control functions include (1) NC part programming, (2) NC part program download, (3) production control, (4) machine control, (5) workpart control, (6) tool management, (7) work transport control, and (8) general system management.

- What are the advantages of FMS technology, compared to conventional batch operations?

Answer. Advantages include (1) higher machine utilization, (2) reduced work-in-process, (3) lower manufacturing lead times, and (4) greater flexibility in production scheduling.

- Define lean production.

- Answer. As defined in the text, lean production is “an adaptation of mass production in which workers and work cells are made more flexible and efficient by adopting methods that reduce waste in all forms.” A shorter definition given in the text is: Lean production is a plant-wide approach aimed at eliminating waste and increasing productivity in manufacturing.

- Identify the principal objective in a just-in-time production system.

- Answer. The principal objective in just-in-time production is reduction of in-process inventory.

- How is a pull system distinguished from a push system in production and inventory control?

- Answer. In a pull system, authorization to produce parts originates from downstream stations. Parts are “ordered” from upstream stations. A push system, by contrast, operates by supplying parts to each station in the plant, in effect driving the work from upstream stations to downstream stations. An MRP system operates as a push system.

- Define computer integrated manufacturing.

Answer. Computer integrated manufacturing (CIM) refers to the pervasive use of computer systems throughout a manufacturing organization, not only to monitor and control the operations, but also to design the product, plan the manufacturing processes, and accomplish the business functions related to production.

# Problems

- A leadscrew with a 7.5 mm pitch drives a worktable in an NC positioning system. The leadscrew is powered by a stepping motor which has 200 step angles. The worktable is programmed to move a distance of 120 mm from its present position at a travel speed of 300 mm/min. Determine (a) the number of pulses required to move the table the specified distance and (b) the required motor speed and pulse rate to achieve the desired table speed.

- Solution: (a)  = 360/ns = 360/200 = 1.8

- np = 360x/p = 360(120)/(7.5 x 1.8) = 3200 pulses

- (b) Nm = vt/p = (300 mm/min)/(7.5 mm/rev) = 40 rev/min

- fp = vtns/60p = 300(200)/(60 x 7.5) = 133.33 Hz

- A stepping motor has 200 step angles. Its output shaft is directly coupled to leadscrew with pitch = 0.250 in. A worktable is driven by the leadscrew. The table must move a distance of 5.00 in from its present position at a travel speed of 20.0 in/min. Determine (a) the number of pulses required to move the table the specified distance and (b) the required motor speed and pulse rate to achieve the specified table speed.

- Solution: (a)  = 360/ns = 360/200 = 1.8

- np = 360x/p = 360(5.0)/(0.25 x 1.8) = 4000 pulses

- (b) Nm = vt/p = (20 in/min)/(0.25 in/rev) = 80 rev/min

- fp = vtns/60p = 20(200)/(60 x 0.25) = 266.67 Hz

- A stepping motor with 100 step angles is coupled to a leadscrew through a gear reduction of 9:1 (9 rotations of the motor for each rotation of the leadscrew). The leadscrew has 5 threads/in. The worktable driven by the leadscrew must move a distance = 10.00 in at a feed rate of 30.0 in/min. Determine (a) number of pulses required to move the table, and (b) the required motor speed and pulse rate to achieve the desired table speed.

- Solution: (a)  = 360/ns = 360/100 = 3.6

- np = 360 rg x/p = 360(9)(10)/(0.2 x 3.6) = 45,000 pulses

- (b) Nm = rg fr/p = 9(30 in/min)/(0.2 in/rev) = 1350 rev/min

- fp = rgfrns/60p = 9(30)(100)/(60 x 0.2) = 2250 Hz

- An NC machine tool table is powered by a servomotor, leadscrew, and optical encoder. The leadscrew has a pitch = 5.0 mm and is connected to the motor shaft with a gear ratio of 16:1 (16 turns of the motor for each turn of the leadscrew). The optical encoder is connected directly to the leadscrew and generates 200 pulses/rev of the leadscrew. The table must move a distance = 100 mm at a feed rate = 500 mm/min. Determine (a) the pulse count received by the control system to verify that the table has moved exactly 100 mm; and (b) the pulse rate and (c) motor speed that correspond to the feed rate of 500 mm/min.

- Solution: (a) x = pnp/ns ; rearranging, np = xns/p = 100(200)/5 = 4000 pulses

- (b) fp = frns/60p = 500(200)/60(5) = 333.3 Hz

- (c) Nm = rgfr/p = 16 x 500/5 = 1600 rev/min

- The worktable of a numerical control machine tool is driven by a closed-loop positioning system which consists of a servomotor, leadscrew, and optical encoder. The leadscrew has 4 threads/in and is coupled directly to the motor shaft (gear ratio = 1:1). The optical encoder generates 200 pulses per motor revolution. The table has been programmed to move a distance of 7.5 in at a feed rate = 20.0 in/min. (a) How many pulses are received by the control system to verify that the table has moved the programmed distance? What are (b) the pulse rate and (c) motor speed that correspond to the specified feed rate?

- Solution: (a) x = pnp/ns ;Rearranging, np = xns/p = 7.5(200)/0.25 = 6000 pulses.

- (b) fp = frns/60p = 20(200)/60(0.25) = 266.67 Hz

- (c) Nm = fr/p = 20/0.25 = 80 rev/min

- A leadscrew coupled directly to a dc servomotor is used to drive one of the table axes of an NC milling machine. The leadscrew has 5 threads/in. The optical encoder attached to the leadscrew emits 100 pulses/rev of the leadscrew. The motor rotates at a maximum speed of 800 rev/min. Determine (a) the frequency of the pulse train emitted by the optical encoder when the servomotor operates at maximum speed and (b) the travel speed of the table at the maximum rpm of the motor.

- Solution: (a) Nm = Nls = 800 rev/min because the motor is connected directly to the leadscrew.

- fp = Nlsns/60 = 800(100)/60 = 1333.3 Hz

- (b) vt = Nls p = 800(0.2) = 160 in/min

- Solve the previous problem only the servomotor is connected to the leadscrew through a gear box whose reduction ratio = 12:1 (12 revolutions of the motor for each revolution of the leadscrew).

- Solution: (a) fp = Nm ns/60rg = 800(100)/60(12) = 111.1 Hz

- (b) vt = Nm p/rg = 800(0.2)/12 = 13.33 in/min

- A leadscrew connected directly to a DC servomotor is the drive system for a positioning table. The leadscrew pitch = 4 mm. The optical encoder attached to the leadscrew emits 250 pulses/rev of the leadscrew. Determine (a) the frequency of the pulse train emitted by the optical encoder when the servomotor operates at 14 rev/s and (b) the travel speed of the table at the operating speed of the motor.

- Solution: (a) Nm = Nls = 14 rev/sec because the motor is connected directly to the leadscrew.

- fp = Nm ns = 14(250) = 3500 Hz

- (b) vt = Nm p = 14(4) = 56 mm/s

- A milling operation is performed on an NC machining center. Total travel distance = 300 mm in a direction parallel to one of the axes of the worktable. Cutting speed = 1.25 m/s and chip load = 0.05 mm. The end milling cutter has four teeth and its diameter = 20.0 mm. The axis uses a DC servomotor whose output shaft is coupled to a leadscrew with pitch = 6.0 mm. The feedback sensing device connected to the leadscrew is an optical encoder that emits 250 pulses per revolution. Determine (a) feed rate and time to complete the cut, and (b) rotational speed of the motor and the pulse rate of the encoder at the feed rate indicated.

- Solution: (a) Spindle speed N = (1.25 x 103 mm/s)/(20 mm/rev) = 19.89 rev/s

- fr = N f nt = 19.89(0.05)(4) = 3.978 mm/s.

- Tm = 300/3.978 = 75.4 s = 1.26 min

- (b) Nm = fr/p = (3.978 mm/s)/(6 mm/rev) = 0.663 rev/s

- fp = ns Nm = 250(0.663) = 165.75 Hz

- An end milling operation is carried out along a straight line path that is 325 mm long. The cut is in a direction parallel to the x-axis on an NC machining center. Cutting speed = 30 m/min and chip load = 0.06 mm. The end milling cutter has two teeth and its diameter = 16.0 mm. The x-axis uses a DC servomotor connected directly to a leadscrew whose pitch = 6.0 mm. The feedback sensing device is an optical encoder that emits 400 pulses per revolution. Determine (a) feed rate and time to complete the cut, and (b) rotational speed of the motor and the pulse rate of the encoder at the feed rate indicated.

Solution: (a) Spindle speed N = (30 x 103 mm/min)/(16 mm/rev) = 596.8 rev/min

fr = N f nt = 596.8(0.06)(2) = 71.62 mm/min

Tm = 325/71.62 = 4.54 min

(b) Nm = fr/p = (71.62 mm/min)/(6.0 mm/rev) = 11.94 rev/min

fp = Nm ns/60 = 400(11.94)/60 = 79.58 Hz

- A DC servomotor drives the x-axis of a NC milling machine table. The motor is coupled to the table lead screw using a 4:1 gear reduction (4 turns of the motor for each turn of the lead screw). The lead screw pitch = 6.25 mm. An optical encoder is connected to the lead screw. The optical encoder emits 500 pulses per revolution. To execute a certain programmed instruction, the table must move from point (x = 87.5 mm, y = 35.0) to point (x = 25.0 mm, y = 180.0 mm) in a straight-line trajectory at a feed rate = 200 mm/min. For the x-axis only, determine (a) the rotational speed of the motor and (b) frequency of the pulse train emitted by the optical encoder at the desired feed rate.

Solution: (a) Move from (87.5, 35.0) to (25.0, 180.0) at fr = 200 mm/min

x = 25.0 - 87.5 = -62.5, y = 180.0 - 35.0 = 145.0, Angle A = tan-1(145/-62.5) = 113.32

frx = 200 cos 113.32 = 200(-0.3958) = -79.19 mm/min

Nm = rg frx/p = 4(-79.17 mm/min)/(6.25 mm/rev) = -50.677 rev/min

(b) fp = nsNls/60 = nsNm/60rg = = 105.58 Hz

- A DC servomotor drives the y-axis of a NC milling machine table. The motor is coupled to the table lead screw with a gear reduction of 2:1 (2 turns of the motor shaft for each single rotation of the lead screw). There are 2 threads per cm in the lead screw. An optical encoder is directly connected to the lead screw (1:1 gear ratio). The optical encoder emits 100 pulses per revolution. To execute a certain programmed instruction, the table must move from point (x = 25.0 mm, y = 28.0) to point (x = 155.0 mm, y = 275.0 mm) in a straight-line trajectory at a feed rate = 200 mm/min. For the y-axis only, determine (a) the rotational speed of the motor and (b) frequency of the pulse train emitted by the optical encoder at the desired feed rate.

Solution: (a) Move from (25, 28) to (155, 275) at 200 mm/min

x = 155 – 25 = 130 mm, y = 275 – 28 = 247 mm

Angle A = tan-1(247/130) = tan-1(1.9) = 62.24

fry = 200 sin 62.24 = 200(0.8849) = 176.98 mm/min

Leadscrew Nlsy = fry/p = 176.98/5 = 35.396 rev/min

Motor Nmy = rgfry/p = 2(176.98)/5 = 70.792 rev/min

(b) Pulse frequency corresponds to rotational speed of leadscrew:

fp = nsNlsy/60 = 100(35.396)/60 = 58.99 Hz